Wireless Dynamics Sensors System (WDSS) with LabVIEW Documentation

Vernier Software & Technology produces data collection software that is used by the majority of our customers using our sensors. We also support “do-it-yourself” projects done with LabVIEW, the graphical programming language from National Instruments. We have sample LabVIEW code on our web site at www.vernier.com/labview to support most of our interfaces. Using the SDK for the WDSS, we have created a library of VIs (short for virtual instruments, what LabVIEW calls programs) that allow LabVIEW to connect, control, and collect data with the WDSS. Included in this package is a sample VIs to introduce LabVIEW programming and working with the WDSS VIs.

These VIs are Windows only. LabVIEW 2009 or later editions are required. A Bluetooth adaptor is needed for connection with the WDSS.

The included VI, WDSS Read and Plot All Sensors, reads all sensors on the WDSS and graphs them all on separate graphs. Use this VI as a model for making your own.

INSTALATION

- Download the zip file “LabVIEW WDSS.zip” from the Vernier website under downloads and LabVIEW VIs.

- Extract the zip file.

- Locate the “WDSS” folder. Place it into the user.lib folder in your LabVIEW installation directory. The default location for the 2009 version is C:\Program Files\National Instruments\LabVIEW 2009\user.lib (for LabVIEW 2010: C:\Program Files\National Instruments\LabVIEW 2010\user.lib)

- If LabVIEW is running, restart it.

- The VIs can now be accessed in LabVIEW by selecting User Libraries on the Function Palette.

The example VI is located in the folder “WDSS example.” User instructions for the example appear on Front Panel of the VI.

Opening up a Bluetooth Connection

Use the Windows Bluetooth device manager and click “Add Device” to find your WDSS. When it asks for a pairing code use “Pair without using a code.” When running a VI, the Bluetooth light on the WDSS should turn green.

An alternative way to connect is by running LoggerPro. This should be done on version 3.8.4 or newer. Go to Experiment>Connect  Interface>Wireless>Scan for Wireless Devices. In the Open Wireless window, select your WDSS and click OK.

For more connection help, visit the Bluetooth help page on the Vernier website at http://www.vernier.com/bthelp/.